Computer Vision Exercise 2

Spatial Intersection and Resection

Yi Wang 3371561 (resubmit)

I. Processing Steps

1. Compute projection matrix & pixel coordinates

Compute projection matrix:

Compute pixel coordinates:

2. Measure one object

3. Spatial intersection

For unknown object coordinate 𝑿 at least two pixel measures 𝒙 and 𝒙′ from two cameras with known projection matrix 𝑷 and 𝑷′are available:

From which we can build identity equation:

With

Then we get

For both x and x’, the third equation is linearly dependent on the other two, therefore we eliminate it and get:

where

And

Solve this equation using singular vector decomposition, we can get the final object coordinates.

4. Back transformation and errors

We apply back transformation with

And calculate the error with

5. Direct Linear Transformation

For the direct linear transformation we use the following equation, where P matrix is what we need.

This can be rewritten into

The third row is linear dependent on the first two rows, therefore we can eliminate it:

To solve this equation we need more than 6 pairs of points:

Similar to before, we use singular vector decomposition to calculate the P matrix.

6. Re-mapping and comparison

Similar to task 1,  is used to re-compute the mapping

7. Reconstruct the camera parameters

a) translation vector X0

𝑿0 can be computed from Singular Value Decomposition (SVD) of P:

U,D,V=svd(P,0)

Where X0 is the last column of V

b) camera matrix K and rotation matrix R

Where 𝑴 = 𝑲𝑹 is the left 3x3-Sub-Matrix of P, With M-1=RTK-1 matrix 𝑴 can be decomposed into QR decomposition:

q,r=qr(M-1)

And

R=q-1

K=r-1

PS: we have to normalize the K and X0 by the scale factor.

II. Results

1. Fundamental matrix & Pixel coordinates

Table1. Fundamental matrix P

| 3497.48715935772 | 2083.00231625386 | 1869.39205798182 | -13101021647.8206 |
| --- | --- | --- | --- |
| 2323.73516206219 | -3332.09572565785 | 997.544343618571 | 16893075551.3581 |
| 0.113062965097000 | -0.0513027902360000 | 0.992262460056000 | 219946.269494256 |

Table2. Pixel coordinates

| ID | 15 | 24 | 25 | 32 | 37 | 98 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| X (pix) | 3182.8948 | 735.9314 | 626.0374 | 3883.0065 | 4277.4454 | 1950.6201 | 1831.1469 |
| Y (pix) | 2309.9545 | 2663.2930 | 236.2401 | 2654.6698 | 235.6924 | 533.9252 | 502.6307 |

Fig1. Object points in image

2. Measure an object

3. Object Coordinates

Table3. Calculated object coordinates

| X (m) | 512997.1910 |
| --- | --- |
| Y (m) | 5427680.3764 |
| Z (m) | 326.5378 |

4. Back transformation errors

Table4. Differences between pixel coordinates of origin points and back transformed points

| Photo_ID | 20813 | 20814 | 20815 | 20816 | 20849 | 20850 | 20851 | 20852 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Diff_X (pix) | 2.8041 | 0.9643 | -6.1380 | 1.6560 | -5.6518 | 2.7380 | 2.8864 | -0.4562 |
| Diff_Y (pix) | 1.0683 | 0.5424 | -0.2438 | -0.8561 | 0.1675 | -2.2750 | 2.8403 | -0.3507 |

Table5. total transformation error

| σx (pix) | 2.7342 |
| --- | --- |
| σy (pix) | 1.0962 |

5. Direct Linear Transformation

Table6. P_DLT (without centralization)

| 0 | 0 | 0 | -0.6128 |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0.7902 |
| 0 | 0 | 0 | 0 |

Table7. P_DLT (after centralization)

| 0.0052 | 0.0031 | 0.0028 | -0.8836 |
| --- | --- | --- | --- |
| 0.0035 | -0.0050 | 0.0015 | -0.4681 |
| 0 | 0 | 0 | -0.0003 |

6. Remapping Difference & Error

Table8. Remapping differences

| ID | 15 | 24 | 25 | 32 | 37 | 98 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| X (pix) | 3.15e-08 | -1.32e-08 | -1.64e-08 | 1.13e-07 | 1.05e-07 | -5.16e-08 | -4.64e-08 |
| Y (pix) | 1.39e-08 | -4.08e-08 | -6.92e-09 | 8.10e-08 | 5.60e-09 | -8.54e-09 | -1.85e-08 |

Table9. Remapping error

| σx (pix) | 5.23e-08 |
| --- | --- |
| σy (pix) | 2.85e-08 |

7. Reconstruct Camera Parameters

a) reconstructed camera parameters

X0=

512980.99515427701.5267514.7943

K=

-3933.36361.29e-082143.500003933.36361423.5000001

R=

-0.827570749775-0.5575304115930.0654713240260.549857636162-0.828569770063-0.1054927300480.113062965097-0.0513027902360.992262460056

b) Differences

X0_difference=

1.59e-081.86e-09-3.23e-08

K_difference=

5.15e-071.29e-08-3.60e-070-5.33e-07-2.07e-07000

R_difference=

-1.58e-124.85e-13-2.06e-112.02e-125.89e-137.57e-12-2.07e-11-5.34e-122.63e-12

III. re-submit remark

The large differences (my first submission) between origin and remap coordinates and camera parameters in task 5,6 and 7 results from the round error of MATLAB, which moreover comes from the large scale difference between object horizon and vertical coordinates (XY has a scale of 106 but Z only 103, this leads to a round error when calculating P matrix). To solve this problem, we could simply centralize the object coordinates at the very beginning by subtracting the mean value (see code file also: four more lines in task 5).

X_20851_m = repmat(mean(X_20851'),7,1)';

X_20851_m(4,:) = 0;

X_20851_c = X_20851-X_20851_m;  % Centralization